During the recharge of the lead storage battery, the reaction that occurs at the anode is

A. PbSO4+2e-→Pb+SO42-
B. PbSO4+2H2O→PbO2+SO42-+4H++2e-
C. Pb+SO42-→PbSO4+2e-
D. PbO2+SO42-+4H++2e-→PbSO4+2H2O
ANSWER: A

During the recharge of the lead storage battery, the reaction that occurs at the cathode is

A. PbSO4+2e-→Pb+SO42-
B. PbSO4+2H2O→PbO2+SO42-+4H++2e-
C. Pb+SO42-→PbSO4+2e-
D. PbO2+SO42-+4H++2e-→PbSO4+2H2O
ANSWER: B

During the discharge of the lead storage battery, the reaction that occurs at the cathode is

A. PbSO4+2e-→Pb+SO42-
B. PbSO4+2H2O→PbO2+SO42-+4H++2e-
C. Pb+SO42-→PbSO4+2e-
D. PbO2+SO42-+4H++2e-→PbSO4+2H2O
ANSWER: D

During the discharge of the lead storage battery, the reaction that occurs at the anode is

A. PbSO4+2e-→Pb+SO42-
B. PbSO4+2H2O→PbO2+SO42-+4H++2e-
C. Pb+SO42-→PbSO4+2e-
D. PbO2+SO42-+4H++2e-→PbSO4+2H2O
ANSWER: C

How the lead storage battery is recharged?
A. By connecting the anode to the negative terminal and the cathode to the positive terminal
B. By connecting the cathode to the negative terminal and the anode to the positive terminal
C. By immersing the lead and lead oxide plates in Sulphuric acid
D. None of these
ANSWER: A

After recharging, It is deposited on the anode of lead storage battery.
A. Pb(s) 
B. PbO2(s)
C. PbSO4(s)
D. PbSO4(aq)
ANSWER: A

After recharging, It is deposited on the cathode of lead storage battery.
A. Pb(s) 
B. PbO2(s)
C. PbSO4(s)
D. PbSO4(aq)
ANSWER: B

What happened to the H2SO4 after the battery gets recharge?
A. H2SO4 solution is used up 
B. H2SO4 solution becomes diluted
C. H2SO4 solution becomes concentrated
D. None of these
ANSWER: C

Active materials of a lead acid cell are

A. Spongy lead

B. Lead peroxide

C. Dilute H2SO4

D. All of the above

ANSWER: D

Sulphation in a lead acid battery occurs due to

A. Heavy charging

B. Fast charging

C. Trickle charging

D. Incomplete charging

ANSWER: D

During the charging of a lead acid cell

A. It's voltage increases

B. It gives out energy

C. It's cathode becomes dark chocolate brown in colour

D. Specific gravity of H2SO4 decreases

ANSWER: A

During charging the specific gravity of the electrolyte of a lead acid battery

A. Increases

B. Decreases

C. Remains the same

D. Becomes zero

ANSWER: A

When a lead acid battery is in fully charged condition the colour of its positive plate is

A. Dark green

B. Brown

C. Dark brown

D. None of the above

ANSWER: C

A battery consists of which type of cells?

A. Electrolytic

B. Electrochemical

C. Electroplating

D. Electromagnetic

ANSWER: B

Given the lead acid battery reaction: Pb + PbO2+H2SO4→2PbSO4+2H2O which species is oxidized during battery discharge?

A. Pb

B. PbO2

C. SO42-

D. 2H2O

ANSWER: A

Which substance functions as electrolyte in automobile battery?

A. PbO2

B. PbSO4

C. H2SO4

D. H2O

ANSWER: C

Give the reaction for the nickel-cadmium battery: 2NiOH+Cd+2H2O→2NiOH2+CdOH2 which species is oxidized during the charge of the battery?

A. Ni3+

B. Ni2+

C. Cd

D. Cd2+

ANSWER: C