The dry cell is the modification of

A. Daniell cell

B. Leclanche cell

C. Lead-acid cell

D. Edison cell

ANSWER: B

All dry cells venue have about

A. 1.0 V

B. 1.2 V

C. 1.5 V

D. 2 V

ANSWER: C

Even though the voltage of a dry cell, big or small, is 1.5 V, a small cell delivers less current because it has comparatively ____ internal resistance

A. Zero

B. Less

C. More

D. Same

ANSWER: C

The internal resistance of a dry cell is of the order of

A. 0.01 to 0.04 ohm

B. 0.2 to 0.5 ohm

C. 2 to 5 ohm

D. 10 to 30 ohm

ANSWER: B

Which of the following has lower specific gravity?

A. Dilute H2SO4

B. Concentrated H2SO4

C. Water

D. Any of the above

ANSWER: C

The anode in the dry cell is
A. Zinc
B. Graphite rod
C. Mixture of MnO2 and charcoal
D. Mixture of NH4Cl and ZnCl2
ANSWER: A

The cathode in the dry cell is

A. Zinc
B. Graphite rod
C. Mixture of MnO2 and charcoal
D. Mixture of NH4Cl and ZnCl2
ANSWER: B

Which of the following reaction occurs at the anode in the dry cell?

A. Zn→Zn2++2e- 
B. 2NH4++2MnO2+2e-→Mn2O3+2NH3+H2O
C. Zn+2OH-→ZnOH2+2e-
D. 2MnO2+H2O+2e-→Mn2O3+2OH-
ANSWER: A

Which of the following reaction occurs at the cathode in the dry cell?

A. Zn→Zn2++2e- 
B. 2NH4++2MnO2+2e-→Mn2O3+2NH3+H2O
C. Zn+2OH-→ZnOH2+2e-
D. 2MnO2+H2O+2e-→Mn2O3+2OH-
ANSWER: B

It is the reaction occurs at the cathode in the alkaline dry cell?

A. Zn→Zn2++2e- 
B. 2NH4++2MnO2+2e-→Mn2O3+2NH3+H2O
C. Zn+2OH-→ZnOH2+2e-
D. 2MnO2+H2O+2e-→Mn2O3+2OH-
ANSWER: D

Which of the following reaction occurs at the anode in the alkaline dry cell?

A. Zn→Zn2++2e- 
B. 2NH4++2MnO2+2e-→Mn2O3+2NH3+H2O
C. Zn+2OH-→ZnOH2+2e-
D. 2MnO2+H2O+2e-→Mn2O3+2OH-
ANSWER: C

Which of the following substance is used as a cathode in NCAD?
A. Ni
B. Cd
C. NiO2
D. Graphite
ANSWER: C

Which of the following substance is used as anode in NCAD?
A. Ni
B. Cd
C. NiO2
D. Graphite
ANSWER: B

With the help of reaction: Zn+2MnO2→ZnO+Mn2O3 Identify the substance reduced in the alkaline dry cell.
A. Zn
B. Mn
C. O
D. None of these
ANSWER: B

With the help of reaction: Zn+2MnO2→ZnO+Mn2O3 Identify the substance oxidized in the alkaline dry cell.
A. Zn
B. Mn
C. O
D. None of these
ANSWER: A

Is calcium oxidized or reduced in the following reaction? 2CaO→2 Ca+O2

A. Oxidized

B. Reduced
ANSWER: 

Which element is being oxidized? HNO3+H2S→NO+S+H2O
A. N
B. O
C. H
D. S
ANSWER: D

In the reaction Zn + 2HCl→ ZnCl2+H2 which element, if any, is oxidized?

A. Zinc

B. Hydrogen

C. Chlorine

D. None
ANSWER: A

Which substance is oxidized? SiO2 + C → Si + CO2
A. Si
B. C
C. O
D. SiO2
ANSWER: B

Which substance is oxidized in the following reaction? Ni+CdOH2→Cd+NiOH2
A. Ni
B. CdOH2
C. Cd
D. NiOH2
ANSWER: A

Which substance is reduced in the following reaction? Ni+CdOH2→Cd+NiOH2
A. Ni
B. CdOH2
C. Cd
D. NiOH2
ANSWER: C

The substance that oxidizes another substance by accepting its electrons.

A. reducing agent

B. oxidation number

C. disproportionation

D. oxidizing agent
ANSWER: D