For the reaction, N2+3H2→2NH3 The production of NH3 will be favored at

A. High pressure and catalyst

B. C. Low pressure and catalyst

C. Low pressure only

D. High pressure only

ANSWER: A

Haber Process is used to produce ammonia commercially according to the following equilibrium:

N2g+3H2g ⇌ 2NH3g + energy

Which of the following conditions will produce the highest yield of ammonia?

A. Increase temperature and increase pressure

B. Increase temperature and decrease pressure

C. Decrease temperature and increase pressure

D. Decrease temperature and decrease pressure

ANSWER: C

In Haber process, equilibrium mixture contains __________ NH3 by volume

A. 20%

B. 55%

C. 70%

D. 35%

ANSWER: D

The catalyst in Haber’s process is pieces of iron crystals embedded in a fused mixture of:

A. MgO

B. Al2O3

C. SiO2

D. All of these

ANSWER: D

Nearly __________ of all nitrogen fixation on earth is accomplished industrially through Haber’s process:

A. 12%

B. 15%

C. 43%

D. 13%

ANSWER: D

Haber’s process synthesis approximately ___________ tons of ammonia

A. 110 million

B. 10 million

C. 1 million

D. 100 million

ANSWER: A

About 80% of ammonia is used for:

A. Production of fertilizers

B. Manufacture of explosives

C. Production of nylon

D. Production of polymer

ANSWER: A

Haber’s process is chemically shown as: N2(g)+3H2(g)⇋2NH3(g) ΔH=-92.46KJmol, what condition will be in favor to obtain more ammonia?

A. Increase the concentration of H2 & N2, high temperature & high pressure

B. Increase the concentration of H2 & N2, low temperature & low pressure

C. Increase the concentration of H2 & N2, high temperature & low pressure

D. Increase the concentration of H2 & N2, low temperature & high pressure

ANSWER: D

Practically, which condition increase the rate of reaction in Contact’s process?

A. high temperature & high pressure

B. low temperature & low pressure

C. low temperature & high pressure

D. high temperature & low pressure

ANSWER: A

Equilibrium mixture in Contact’s process contains:

A. 25% ammonia & 75% Nitrogen and Hydrogen gas

B. 35% ammonia & 65% Nitrogen and Hydrogen gas

C. 15% ammonia & 85% Nitrogen and Hydrogen gas

D. 45% ammonia & 55% Nitrogen and Hydrogen gas

ANSWER: B

Contact’s process is chemically shown as: SO2(g)+2O2(g)⇋2SO3(g) ΔH=-194 KJmol, what condition will be in favor to obtain more SO3?

A. Increase the concentration of O2 & SO2, high temperature & high pressure

B. Increase the concentration of O2 & SO2, low temperature & low pressure

C. Increase the concentration of O2 & SO2, high temperature & low pressure

D. Increase the concentration of O2 & SO2, low temperature & high pressure

ANSWER: D

Practically, which condition increase the rate of reaction in Contact’s process?

A. high temperature & high pressure

B. low temperature & low pressure

C. low temperature & high pressure

D. high temperature & low pressure

ANSWER: D

The equilibrium between water and its vapour in an open vessel

A. can be achieved

B. depends upon pressure

C. cannot be achieved

D. depends upon temperature

ANSWER: C

When a catalyst is added to a reversible reaction in equilibrium state the value of the equilibrium constant

A. increases

B. decreases

C. does not change

D. becomes zero

ANSWER: C

A vessel at equilibrium contain SO3, SO2 and O2, now some helium gas is added so that total pressure increases while temperature and volume remain constant. According to Le Chatelier Principle the dissociation of SO32-

A. decreases

B. remain constant

C. increases

D. change unpredictably

ANSWER: B